DEVELOPEMENT UTILITIES

======================

RELEASE V1.0

DATE    17th. AUGUST 1987

DISCLAIMER

----------

This is not a Psion supported product and as such is used entirely at

the users own risk. At no time will Psion be responsible for maintaining

this product in any way, and may indeed deny any knowledge of this

product in the future.

It is expected that users of this product will have the technical manual

and be fully conversant with 6303X machine code programming.

However if you have a problem, and that problem is not how do I program

in machine code, we are prepared to advise to you. However if you abuse

this privilege we will be forced to charge you 50 pounds an hour or

part thereof.

This product may be copied and distributed free to anyone on condition

that the above disclaimer is also included.

Colly Myers (Development Director, Psion).

------------------------------------------

Files included on this disk:

----------------------------

XA.EXE		Assembler

XA.MAN		Assembler manual

DBORG.EXE	Organiser debugger

DBORG.MAN	Organiser debugger manual

DBBOOT.BIN	Binary file used by DBORG

DBMAIN.BIN	Binary file used by DBORG

OSHEAD.INC	The main header, includes macros, equ's and #defines

SWI.INC		Include all the SWI's

OSVARS.INC	Includes all the operating systems variables

OSEROR.INC	Includes all the operating system errors

DICT.SRC	Example of assembler code

XDICT.SRC	As DICT but with a 6 byte header to make it a .BIN

BOOT.SRC	The source code for making a pack bootable

README.DOC	This document

DICT.SRC & XDICT.SRC

====================

DICT.SRC is included on the disk as an example of an assembler source file.

It consists of the assembler code that is used on the CONCISE OXFORD SPELLING

CHECKER pack (the compressed data is not included).

You should note the 10 header bytes and the 11 bytes for the MAIN file name

that are at the start; all packs should have these to be in standard pack

format.  When this code is built into a product certain of the first 10

bytes are liable to be changed:

byte 0		Bits in the mask ^xC7 are libale to be changed

depending on the hardware.  You can determine if the

pack is bootable, copy protected and write protected.

Note that the last two can be overwritten in BLDPACK.

byte 1		Depends on the size of the pack.

byte 8 & 9	Used as checksum word on RAMPACKS.

The only difference between DICT.SRC and XDICT.SRC is that XDICT has an

extra 6 byte header which, when assembled with the '-b' flag, allows the

resulting .BIN to be used by BLDPACK and then MAKE to produce a pack with

the code on.  The 6 bytes consist of the letters 'O', 'R' and 'G', a 2 byte

length with the most significant byte at the lowest address and finally ^xFF.

Assembler .BINs may not be bigger than 64K.

BOOT.SRC

========

BOOT.SRC is the assembler source code used to make a pack bootable.

It already has the 6 byte header required to make it into a .bin. This

is the same as the BOOT.BIN supplied with the ORGANISER DEVELOPER.